ARSH

23K-0078
BAI-3C

LAB 4

# Task 1

Declare num1 = 1500h and num2 = 2500h, copy them through EAX/EBX/ECX, swap EBX and ECX with XCHG, store the results in memory and display the registers with DumpRegs.

## Code

INCLUDE Irvine32.inc

.data

num1        DWORD 1500h

num2        DWORD 2500h

finalEBX    DWORD ?

finalECX    DWORD ?

msgTitle    BYTE "Task 1 - MOV and XCHG", 0

msgEBX      BYTE "finalEBX (memory) = ", 0

msgECX      BYTE "finalECX (memory) = ", 0

msgHex      BYTE "h", 0

.code

main PROC

mov     edx, OFFSET msgTitle

call    WriteString

call    Crlf

mov     eax, num1           ; EAX = 00001500h

mov     ebx, eax            ; EBX = 00001500h

mov     ecx, num2           ; ECX = 00002500h

xchg    ebx, ecx            ; EBX = 00002500h, ECX = 00001500h

mov     finalEBX, ebx       ; store final EBX in memory

mov     finalECX, ecx       ; store final ECX in memory

call    DumpRegs            ; show EAX, EBX, ECX

; display both memory variables

mov     edx, OFFSET msgEBX

call    WriteString

mov     eax, finalEBX

call    WriteHex

mov     edx, OFFSET msgHex

call    WriteString

call    Crlf

mov     edx, OFFSET msgECX

call    WriteString

mov     eax, finalECX

call    WriteHex

mov     edx, OFFSET msgHex

call    WriteString

call    Crlf

exit

main ENDP

END main

# Task 2

Perform ADD, DEC and NEG on 8-bit registers and record the flags immediately after each instruction (DumpRegs is called right after each operation).

## Code

INCLUDE Irvine32.inc

.data

resAL       BYTE ?

resBL       BYTE ?

resCL       BYTE ?

msgAdd      BYTE "1) MOV AL,7Fh   ADD AL,01h", 0

msgDec      BYTE "2) MOV BL,00h   DEC BL", 0

msgNeg      BYTE "3) MOV CL,05h   NEG CL", 0

msgFinal    BYTE "Final 8-bit register values:", 0

msgAL       BYTE "AL = ", 0

msgBL       BYTE "BL = ", 0

msgCL       BYTE "CL = ", 0

msgHex      BYTE "h", 0

.code

main PROC

xor     eax, eax            ; clear registers so DumpRegs is easy to read

xor     ebx, ebx

xor     ecx, ecx

; ---------- Operation 1 : ADD ----------

mov     edx, OFFSET msgAdd

call    WriteString

mov     al, 7Fh             ; AL = 7Fh (+127)

add     al, 01h             ; AL = 80h  -> CF=0 ZF=0 SF=1 OF=1

call    DumpRegs            ; flags recorded immediately after ADD

mov     resAL, al

; ---------- Operation 2 : DEC ----------

mov     edx, OFFSET msgDec

call    WriteString

mov     bl, 00h             ; BL = 00h

clc                         ; DEC does not touch CF, so start it from a known 0

dec     bl                  ; BL = FFh  -> CF=0 (unchanged) ZF=0 SF=1

call    DumpRegs            ; flags recorded immediately after DEC

mov     resBL, bl

; ---------- Operation 3 : NEG ----------

mov     edx, OFFSET msgNeg

call    WriteString

mov     cl, 05h             ; CL = 05h

neg     cl                  ; CL = FBh (-5) -> CF=1 ZF=0 SF=1 OF=0

call    DumpRegs            ; flags recorded immediately after NEG

mov     resCL, cl

; ---------- Final register values ----------

mov     edx, OFFSET msgFinal

call    WriteString

call    Crlf

mov     ebx, 1              ; WriteHexB: display 1 byte

mov     edx, OFFSET msgAL

call    WriteString

movzx   eax, resAL

call    WriteHexB

mov     edx, OFFSET msgHex

call    WriteString

call    Crlf

mov     edx, OFFSET msgBL

call    WriteString

movzx   eax, resBL

call    WriteHexB

mov     edx, OFFSET msgHex

call    WriteString

call    Crlf

mov     edx, OFFSET msgCL

call    WriteString

movzx   eax, resCL

call    WriteHexB

mov     edx, OFFSET msgHex

call    WriteString

call    Crlf

exit

main ENDP

END main

## Flag Table 1: MOV AL, 7Fh / ADD AL, 01h  →  AL = 80h

| Flag | Value | Why |
| --- | --- | --- |
| CF | 0 | 7Fh + 01h = 80h fits in 8 bits, no carry out of bit 7 |
| ZF | 0 | Result 80h is not zero |
| SF | 1 | Bit 7 of 80h (1000 0000b) is 1 |
| OF | 1 | +127 + 1 = +128 is outside the signed byte range (−128 to +127); two positives gave a "negative" result |

## Flag Table 2: MOV BL, 00h / DEC BL  →  BL = FFh

| Flag | Value | Why |
| --- | --- | --- |
| CF | 0 | DEC never changes CF; it keeps its previous value (cleared with CLC before DEC) |
| ZF | 0 | Result FFh is not zero |
| SF | 1 | Bit 7 of FFh is 1 (FFh = −1 signed) |

## Flag Table 3: MOV CL, 05h / NEG CL  →  CL = FBh

| Flag | Value | Why |
| --- | --- | --- |
| CF | 1 | NEG sets CF = 1 whenever the operand is not zero (0 − 5 needs a borrow) |
| ZF | 0 | Result FBh is not zero |
| SF | 1 | FBh = 1111 1011b, bit 7 is 1 (−5 in two's complement) |
| OF | 0 | OF is set by NEG only when the operand is −128 (80h) |

## Final Register Values

| Register | Hex | Unsigned | Signed |
| --- | --- | --- | --- |
| AL | 80h | 128 | −128 |
| BL | FFh | 255 | −1 |
| CL | FBh | 251 | −5 |

# Task 3

Use MOVZX and MOVSX on uVal = 9Bh, sVal = −101 and CL = 9Bh, store the extended values in DWORD variables and display them.

## Code

INCLUDE Irvine32.inc

.data

uVal        BYTE  9Bh           ; unsigned byte

sVal        SBYTE -101          ; signed byte (stored as 9Bh)

zxResult    DWORD ?             ; zero-extended CL

sxResult    DWORD ?             ; sign-extended CL

msgTitle    BYTE "Task 3 - MOVZX / MOVSX", 0

msgEAX      BYTE "EAX = MOVZX uVal  : ", 0

msgEBX      BYTE "EBX = MOVSX sVal  : ", 0

msgZX       BYTE "zxResult (MOVZX CL): ", 0

msgSX       BYTE "sxResult (MOVSX CL): ", 0

msgHex      BYTE "h   decimal = ", 0

.code

main PROC

mov     edx, OFFSET msgTitle

call    WriteString

call    Crlf

movzx   eax, uVal           ; EAX = 0000009Bh = 155

movsx   ebx, sVal           ; EBX = FFFFFF9Bh = -101

xor     ecx, ecx            ; clear ECX so CL is easy to see

mov     cl, 9Bh             ; same bit pattern 1001 1011

movzx   edx, cl             ; EDX = 0000009Bh (zero extension)

movsx   esi, cl             ; ESI = FFFFFF9Bh (sign extension)

mov     zxResult, edx

mov     sxResult, esi

call    DumpRegs

; ---- display every converted value in hex and decimal ----

mov     edx, OFFSET msgEAX

call    WriteString

call    WriteHex            ; EAX already holds MOVZX uVal

mov     edx, OFFSET msgHex

call    WriteString

call    WriteDec            ; unsigned decimal

call    Crlf

mov     edx, OFFSET msgEBX

call    WriteString

mov     eax, ebx

call    WriteHex

mov     edx, OFFSET msgHex

call    WriteString

call    WriteInt            ; signed decimal

call    Crlf

mov     edx, OFFSET msgZX

call    WriteString

mov     eax, zxResult

call    WriteHex

mov     edx, OFFSET msgHex

call    WriteString

call    WriteDec

call    Crlf

mov     edx, OFFSET msgSX

call    WriteString

mov     eax, sxResult

call    WriteHex

mov     edx, OFFSET msgHex

call    WriteString

call    WriteInt

call    Crlf

exit

main ENDP

END main

## Converted Values

| Instruction | Destination | Hex | Decimal |
| --- | --- | --- | --- |
| MOVZX EAX, uVal | EAX | 0000009Bh | 155 |
| MOVSX EBX, sVal | EBX | FFFFFF9Bh | −101 |
| MOVZX EDX, CL | EDX → zxResult | 0000009Bh | 155 |
| MOVSX ESI, CL | ESI → sxResult | FFFFFF9Bh | −101 |

## Explanation

uVal, sVal and CL all hold exactly the same 8 bits: 9Bh = 1001 1011b. The CPU does not know whether a byte is signed or unsigned; the instruction decides how it is interpreted.

- Unsigned: 1001 1011b = 128 + 16 + 8 + 2 + 1 = 155.

- Signed (two's complement): bit 7 is the sign bit, so the value is 155 − 256 = −101.

- MOVZX fills the upper 24 bits with 0, so the number stays 155 (0000009Bh).

- MOVSX copies the sign bit (1) into the upper 24 bits, so the number stays −101 (FFFFFF9Bh).

So the same bit pattern represents 155 when treated as unsigned and −101 when treated as signed.

# Task 4

Use ROWS, COLS and TOTAL to define the values array, find its size with LENGTHOF/SIZEOF, access elements by offset, swap the first and last elements and sum the first four.

## Code

INCLUDE Irvine32.inc

ROWS = 3

COLS = 4

TOTAL EQU ROWS * COLS

.data

values  WORD TOTAL DUP(5)

result  DWORD ?

buffer  BYTE 16 DUP(?)

msgTotal    BYTE "TOTAL (ROWS*COLS)       = ", 0

msgLen      BYTE "LENGTHOF values         = ", 0

msgSize     BYTE "SIZEOF values (bytes)   = ", 0

msgType     BYTE "TYPE values (bytes)     = ", 0

msgBefore   BYTE "--- Before XCHG ---", 0

msgAfter    BYTE "--- After XCHG (first <-> last) ---", 0

msgFirst    BYTE "values[0]  (offset  0) = ", 0

msgFourth   BYTE "values[3]  (offset  6) = ", 0

msgFifth    BYTE "values[4]  (offset  8) = ", 0

msgLast     BYTE "values[11] (offset 22) = ", 0

msgSum      BYTE "Sum of first four elements (result) = ", 0

.code

main PROC

; ---- array dimensions ----

mov     edx, OFFSET msgTotal

call    WriteString

mov     eax, TOTAL                  ; 12

call    WriteDec

call    Crlf

mov     edx, OFFSET msgLen

call    WriteString

mov     eax, LENGTHOF values        ; 12 elements

call    WriteDec

call    Crlf

mov     edx, OFFSET msgSize

call    WriteString

mov     eax, SIZEOF values          ; 12 * 2 = 24 bytes

call    WriteDec

call    Crlf

mov     edx, OFFSET msgType

call    WriteString

mov     eax, TYPE values            ; 2 bytes per element

call    WriteDec

call    Crlf

call    Crlf

; ---- selected elements before the exchange ----

mov     edx, OFFSET msgBefore

call    WriteString

call    Crlf

call    ShowElements

; ---- exchange first and last element ----

mov     ax, values                              ; AX = first element

xchg    ax, [values + SIZEOF values - TYPE values]  ; AX <-> last element

mov     values, ax                              ; old last -> first

mov     edx, OFFSET msgAfter

call    WriteString

call    Crlf

call    ShowElements

; ---- sum of the first four elements ----

movzx   eax, values                 ; values[0]

movzx   ebx, [values + 2]           ; values[1]

add     eax, ebx

movzx   ebx, [values + 4]           ; values[2]

add     eax, ebx

movzx   ebx, [values + 6]           ; values[3]

add     eax, ebx

mov     result, eax                 ; result = 20

mov     edx, OFFSET msgSum

call    WriteString

mov     eax, result

call    WriteDec

call    Crlf

exit

main ENDP

;------------------------------------------------------------

; ShowElements

; Displays the first, fourth, fifth and last elements of values.

;------------------------------------------------------------

ShowElements PROC

mov     edx, OFFSET msgFirst

call    WriteString

movzx   eax, values                 ; offset 0

call    WriteDec

call    Crlf

mov     edx, OFFSET msgFourth

call    WriteString

movzx   eax, [values + 3 * TYPE values]   ; offset 6

call    WriteDec

call    Crlf

mov     edx, OFFSET msgFifth

call    WriteString

movzx   eax, [values + 4 * TYPE values]   ; offset 8

call    WriteDec

call    Crlf

mov     edx, OFFSET msgLast

call    WriteString

movzx   eax, [values + SIZEOF values - TYPE values]   ; offset 22

call    WriteDec

call    Crlf

call    Crlf

ret

ShowElements ENDP

END main

## Calculations

| Item | Calculation | Result |
| --- | --- | --- |
| Total elements (TOTAL / LENGTHOF) | ROWS × COLS = 3 × 4 | 12 |
| Size of one element (TYPE) | WORD | 2 bytes |
| Total bytes (SIZEOF) | 12 × 2 | 24 bytes |
| result | 5 + 5 + 5 + 5 | 20 |

## Element Offsets

| Element | Index | Offset | Operand used | Before XCHG | After XCHG |
| --- | --- | --- | --- | --- | --- |
| First | 0 | 0 | [values] | 5 | 5 |
| Fourth | 3 | 6 | [values + 3*TYPE values] | 5 | 5 |
| Fifth | 4 | 8 | [values + 4*TYPE values] | 5 | 5 |
| Last | 11 | 22 | [values + SIZEOF values − TYPE values] | 5 | 5 |

The exchange is done with AX as the intermediate register (MOV AX, first → XCHG AX, last → MOV first, AX). Because the array is declared as TOTAL DUP(5), every element is 5, so the first and last elements still show 5 after the swap even though the exchange did execute.

## = versus EQU

- = creates an integer symbolic constant that can be redefined later in the program (e.g. ROWS = 5 further down is allowed).

- EQU creates a symbol that cannot be redefined, and it can also stand for text, not only integers.

## Why values reserves memory but TOTAL does not

TOTAL is an assemble-time constant: the assembler simply replaces the name TOTAL with 12 wherever it appears, so it never exists in the program's memory. values is a data definition in the .data segment, so the assembler allocates 12 WORDs (24 bytes) of real storage for it that the program can read and write at run time.

# Task 5

Evaluate R = −(2X − Y) + (Z − 3W) using signed 32-bit variables X = 120, Y = 75, Z = 40, W = 30, without modifying the original variables.

## Code

INCLUDE Irvine32.inc

.data

X           SDWORD 120

Y           SDWORD 75

Z           SDWORD 40

W           SDWORD 30

R           SDWORD ?

msgT1       BYTE "Term 1 : EAX = -(2X - Y)", 0

msgT2       BYTE "Term 2 : ECX = (Z - 3W)", 0

msgFinal    BYTE "Final  : EAX = EAX + ECX  (flags after final ADD)", 0

msgRHex     BYTE "R (hexadecimal)          = ", 0

msgRDec     BYTE "R (signed decimal)       = ", 0

msgVerify   BYTE "Verify: ESI = R          = ", 0

msgUns      BYTE "Same bits read unsigned  = ", 0

msgHex      BYTE "h", 0

.code

main PROC

xor     ebx, ebx                ; clear registers so DumpRegs is easy to read

xor     ecx, ecx

xor     esi, esi

; ---------- Term 1 : -(2X - Y) ----------

mov     edx, OFFSET msgT1

call    WriteString

mov     eax, X                  ; EAX = 120

add     eax, X                  ; EAX = 2X = 240

sub     eax, Y                  ; EAX = 2X - Y = 165

neg     eax                     ; EAX = -165 (FFFFFF5Bh)

call    DumpRegs                ; flags after NEG

; ---------- Term 2 : (Z - 3W) ----------

mov     edx, OFFSET msgT2

call    WriteString

mov     ebx, W                  ; EBX = 30

add     ebx, W                  ; EBX = 60

add     ebx, W                  ; EBX = 3W = 90

mov     ecx, Z                  ; ECX = 40

sub     ecx, ebx                ; ECX = Z - 3W = -50 (FFFFFFCEh)

call    DumpRegs                ; flags after SUB

; ---------- Combine ----------

mov     edx, OFFSET msgFinal

call    WriteString

add     eax, ecx                ; EAX = -165 + (-50) = -215 (FFFFFF29h)

call    DumpRegs                ; flags immediately after the final ADD

mov     R, eax                  ; store final answer

; ---------- Display R ----------

mov     edx, OFFSET msgRHex

call    WriteString

mov     eax, R

call    WriteHex

mov     edx, OFFSET msgHex

call    WriteString

call    Crlf

mov     edx, OFFSET msgRDec

call    WriteString

mov     eax, R

call    WriteInt

call    Crlf

; ---------- Verify by loading R into another register ----------

mov     esi, R                  ; ESI = FFFFFF29h

mov     edx, OFFSET msgVerify

call    WriteString

mov     eax, esi

call    WriteInt                ; signed view   : -215

call    Crlf

mov     edx, OFFSET msgUns

call    WriteString

mov     eax, esi

call    WriteDec                ; unsigned view : 4294967081

call    Crlf

exit

main ENDP

END main

## Intermediate Register Values

| Step | Instruction(s) | Register | Hex | Decimal |
| --- | --- | --- | --- | --- |
| 2X | MOV EAX, X / ADD EAX, X | EAX | 000000F0h | 240 |
| 2X − Y | SUB EAX, Y | EAX | 000000A5h | 165 |
| −(2X − Y) | NEG EAX | EAX | FFFFFF5Bh | −165 |
| 3W | MOV EBX, W / ADD EBX, W (×2) | EBX | 0000005Ah | 90 |
| Z − 3W | MOV ECX, Z / SUB ECX, EBX | ECX | FFFFFFCEh | −50 |
| R | ADD EAX, ECX / MOV R, EAX | EAX → R | FFFFFF29h | −215 |

Check: −(2·120 − 75) + (40 − 3·30) = −165 + (−50) = −215.

## Flags After the Final ADD (EAX = EAX + ECX)

| Flag | Value | Explanation |
| --- | --- | --- |
| CF | 1 | As unsigned numbers FFFFFF5Bh + FFFFFFCEh = 1FFFFFF29h, which carries out of bit 31. CF only matters for unsigned math, so it is ignored for this signed result. |
| ZF | 0 | The result FFFFFF29h is not zero. |
| SF | 1 | Bit 31 of the result is 1, so the signed result is negative. |
| OF | 0 | Two negative numbers were added and the result is also negative; −215 fits in a signed 32-bit integer, so there is no signed overflow. |

## Why R Is a Signed Integer

R is declared as SDWORD and the expression contains negation and subtraction, so the answer can be negative. FFFFFF29h has its most significant bit set, which in two's complement means a negative number: −(100000000h − FFFFFF29h) = −D7h = −215.

To verify, R is loaded into ESI (MOV ESI, R). Printing ESI with WriteInt (signed) gives −215, which matches the hand calculation. Printing the same bits with WriteDec (unsigned) gives 4,294,967,081, which is meaningless for this expression; this shows R must be interpreted as signed.