# ARSH

# 23K-0078
BAI-3C

# LAB 7

# Task 1

Set EAX = 11, EBX = 22, ECX = 33, push them in that order, then pop into EDX, ESI and EDI. Record ESP after every PUSH and POP and display the registers with DumpRegs.

## Prediction (before running)

The stack is Last-In-First-Out, so the last value pushed (ECX = 33) is popped first. Predicted: EDX = 33, ESI = 22, EDI = 11.

## Code

INCLUDE Irvine32.inc

.data

esp0        DWORD ?                     ; ESP before any PUSH

esp1        DWORD ?                     ; after PUSH EAX

esp2        DWORD ?                     ; after PUSH EBX

esp3        DWORD ?                     ; after PUSH ECX

esp4        DWORD ?                     ; after POP EDX

esp5        DWORD ?                     ; after POP ESI

esp6        DWORD ?                     ; after POP EDI

msgRegs     BYTE "Registers after the three POPs:", 0

msgTitle    BYTE "ESP trace (change is relative to the starting ESP):", 0

msgS0       BYTE "Start          ESP = ", 0

msgS1       BYTE "After PUSH EAX ESP = ", 0

msgS2       BYTE "After PUSH EBX ESP = ", 0

msgS3       BYTE "After PUSH ECX ESP = ", 0

msgS4       BYTE "After POP  EDX ESP = ", 0

msgS5       BYTE "After POP  ESI ESP = ", 0

msgS6       BYTE "After POP  EDI ESP = ", 0

msgChange   BYTE "h   change = ", 0

.code

main PROC

mov     edx, OFFSET msgRegs         ; label printed first so EDX is free later

call    WriteString

mov     eax, 11

mov     ebx, 22

mov     ecx, 33

xor     edx, edx                    ; clear destination registers

xor     esi, esi

xor     edi, edi

mov     esp0, esp

push    eax                         ; stack: 11

mov     esp1, esp

push    ebx                         ; stack: 22, 11

mov     esp2, esp

push    ecx                         ; stack: 33, 22, 11   (33 on top)

mov     esp3, esp

pop     edx                         ; EDX = 33 (top of stack)

mov     esp4, esp

pop     esi                         ; ESI = 22

mov     esp5, esp

pop     edi                         ; EDI = 11

mov     esp6, esp

call    DumpRegs                    ; EDX = 33, ESI = 22, EDI = 11

; ---------- ESP trace ----------

mov     edx, OFFSET msgTitle

call    WriteString

call    Crlf

mov     edx, OFFSET msgS0

mov     eax, esp0

call    ShowStep

mov     edx, OFFSET msgS1

mov     eax, esp1

call    ShowStep

mov     edx, OFFSET msgS2

mov     eax, esp2

call    ShowStep

mov     edx, OFFSET msgS3

mov     eax, esp3

call    ShowStep

mov     edx, OFFSET msgS4

mov     eax, esp4

call    ShowStep

mov     edx, OFFSET msgS5

mov     eax, esp5

call    ShowStep

mov     edx, OFFSET msgS6

mov     eax, esp6

call    ShowStep

exit

main ENDP

;------------------------------------------------------------

; ShowStep

; Prints a label, an ESP value and its change from the start.

; Receives: EDX = OFFSET label, EAX = recorded ESP value

;------------------------------------------------------------

ShowStep PROC

call    WriteString

call    WriteHex

mov     edx, OFFSET msgChange

call    WriteString

sub     eax, esp0                   ; change in bytes

call    WriteInt

call    Crlf

ret

ShowStep ENDP

END main

## Output

Result: EDX = 00000021h (33), ESI = 00000016h (22), EDI = 0000000Bh (11), exactly as predicted.

## ESP After Each Operation

| Instruction | ESP (relative) | Top of stack | Stack contents (top first) |
| --- | --- | --- | --- |
| Start | X | — | empty |
| PUSH EAX | X − 4 | 11 | 11 |
| PUSH EBX | X − 8 | 22 | 22, 11 |
| PUSH ECX | X − 12 | 33 | 33, 22, 11 |
| POP EDX | X − 8 | 22 | 22, 11 (EDX = 33) |
| POP ESI | X − 4 | 11 | 11 (ESI = 22) |
| POP EDI | X | — | empty (EDI = 11) |

## Why the Order Is Reversed

PUSH decrements ESP by 4 and then stores the value, so each new value sits on top of the previous one. POP always removes the value at the top (the one ESP points to) and then adds 4 to ESP. Because the stack is LIFO, the values come back in the reverse order: 33, 22, 11. ESP returns to its starting value after the three POPs, so the stack is balanced.

# Task 2

Save EAX = 12345678h and EBX = 0ABCDEF0h on the stack, overwrite them, restore them by popping in reverse order, and check that ESP returns to its original value.

## Code

INCLUDE Irvine32.inc

.data

espBefore   DWORD ?

espAfter    DWORD ?

msgOrig     BYTE "1) Original values (before PUSH):", 0

msgOver     BYTE "2) After PUSH and overwriting EAX/EBX:", 0

msgRest     BYTE "3) After POP EBX, POP EAX (restored):", 0

msgB        BYTE "ESP before the two PUSHes = ", 0

msgA        BYTE "ESP after the two POPs    = ", 0

msgOK       BYTE "Stack pointer is back to its original value.", 0

msgHex      BYTE "h", 0

.code

main PROC

mov     edx, OFFSET msgOrig

call    WriteString

mov     eax, 12345678h

mov     ebx, 0ABCDEF0h

call    DumpRegs

mov     edx, OFFSET msgOver

call    WriteString

mov     espBefore, esp

push    eax                         ; save EAX

push    ebx                         ; save EBX (now on top)

mov     eax, 11111111h              ; overwrite both registers

mov     ebx, 22222222h

call    DumpRegs                    ; ESP is 8 bytes lower here

mov     edx, OFFSET msgRest

call    WriteString

pop     ebx                         ; reverse order: last pushed, first popped

pop     eax

mov     espAfter, esp

call    DumpRegs                    ; EAX = 12345678h, EBX = 0ABCDEF0h

; ---------- show stack balance ----------

mov     edx, OFFSET msgB

call    WriteString

mov     eax, espBefore

call    WriteHex

mov     edx, OFFSET msgHex

call    WriteString

call    Crlf

mov     edx, OFFSET msgA

call    WriteString

mov     eax, espAfter

call    WriteHex

mov     edx, OFFSET msgHex

call    WriteString

call    Crlf

mov     eax, espAfter

cmp     eax, espBefore

jne     Done

mov     edx, OFFSET msgOK

call    WriteString

call    Crlf

Done:

exit

main ENDP

END main

## Output

## Results

| Step | EAX | EBX | ESP |
| --- | --- | --- | --- |
| 1) Original | 12345678h | 0ABCDEF0h | X |
| 2) After PUSH EAX, PUSH EBX and overwrite | 11111111h | 22222222h | X − 8 |
| 3) After POP EBX, POP EAX | 12345678h | 0ABCDEF0h | X |

EBX was pushed last, so it is on top of the stack and must be popped first (POP EBX, then POP EAX). Popping in the same order as the pushes would swap the two values. The DumpRegs output confirms both registers are restored, and the program confirms ESP after the two POPs equals ESP before the two PUSHes.

# Task 3

Create a known flag state, save EFLAGS with PUSHFD, change the flags with other instructions, then restore them with POPFD and inspect the results with DumpRegs.

## Code

INCLUDE Irvine32.inc

.data

msgKnown    BYTE "1) Known state: MOV EAX,5 / CMP EAX,5  (ZF should be 1)", 0

msgChanged  BYTE "2) After PUSHFD and flag-changing instructions", 0

msgRestored BYTE "3) After POPFD (flags restored, registers NOT restored)", 0

.code

main PROC

xor     ebx, ebx                    ; clear registers so DumpRegs is easy to read

xor     ecx, ecx

mov     edx, OFFSET msgKnown

call    WriteString

mov     eax, 5

cmp     eax, 5                      ; 5 - 5 = 0  -> ZF=1, CF=0, SF=0

call    DumpRegs

pushfd                              ; save EFLAGS (ZF=1) on the stack

mov     edx, OFFSET msgChanged

call    WriteString

mov     ebx, 10

add     ebx, 1                      ; EBX = 11, result non-zero -> ZF=0

sub     eax, 3                      ; EAX = 2

mov     ecx, 1

sub     ecx, 2                      ; ECX = FFFFFFFFh -> CF=1, SF=1, ZF=0

call    DumpRegs

mov     edx, OFFSET msgRestored

call    WriteString

popfd                               ; restore EFLAGS saved by PUSHFD

call    DumpRegs                    ; ZF=1 again, but EAX/EBX/ECX keep new values

exit

main ENDP

END main

## Output

## Flag and Register States

| Point | EAX | EBX | ECX | CF | SF | ZF |
| --- | --- | --- | --- | --- | --- | --- |
| 1) After CMP EAX, 5 | 5 | 0 | 0 | 0 | 0 | 1 |
| 2) After the changing instructions | 2 | 0Bh | FFFFFFFFh | 1 | 1 | 0 |
| 3) After POPFD | 2 | 0Bh | FFFFFFFFh | 0 | 0 | 1 |

- ZF before saving: CMP EAX, 5 subtracts 5 − 5 internally. The result is 0, so ZF = 1 (and CF = 0, SF = 0). PUSHFD then saves this EFLAGS value on the stack.

- Changing the flags: ADD EBX, 1 gives a non-zero result (ZF = 0), and SUB ECX, 2 with ECX = 1 needs a borrow, so CF = 1 and SF = 1.

- After POPFD: the flags are exactly the saved ones again (ZF = 1, CF = 0, SF = 0, EFL = 00000246h), but EAX, EBX and ECX keep their new values. POPFD only restores EFLAGS, not general-purpose registers.

There is exactly one PUSHFD and one POPFD, so the stack is balanced (ESP in step 3 equals ESP in step 1).

# Task 4

Create a procedure DisplayStudentInfo that prints a student's name, registration number and course title, and call it three times from main with different information.

## Code

INCLUDE Irvine32.inc

.data

; ----- student 1 -----

name1       BYTE "Ali Ahmed", 0

reg1        BYTE "24K-0512", 0

course1     BYTE "Computer Organization and Assembly Language", 0

; ----- student 2 -----

name2       BYTE "Sara Khan", 0

reg2        BYTE "24K-0738", 0

course2     BYTE "Data Structures", 0

; ----- student 3 -----

name3       BYTE "Usman Raza", 0

reg3        BYTE "24K-0941", 0

course3     BYTE "Digital Logic Design", 0

msgName     BYTE "Student Name    : ", 0

msgReg      BYTE "Registration No : ", 0

msgCourse   BYTE "Course Title    : ", 0

.code

main PROC

mov     esi, OFFSET name1           ; pass the three strings in registers

mov     edi, OFFSET reg1

mov     ebx, OFFSET course1

call    DisplayStudentInfo          ; returns to the next instruction

mov     esi, OFFSET name2

mov     edi, OFFSET reg2

mov     ebx, OFFSET course2

call    DisplayStudentInfo

mov     esi, OFFSET name3

mov     edi, OFFSET reg3

mov     ebx, OFFSET course3

call    DisplayStudentInfo

exit

main ENDP

;------------------------------------------------------------

; DisplayStudentInfo

; Prints a student's name, registration number and course

; title on separate lines, followed by a blank line.

; Receives: ESI = OFFSET name, EDI = OFFSET reg. no,

;           EBX = OFFSET course title

; Returns:  nothing (EDX is preserved)

;------------------------------------------------------------

DisplayStudentInfo PROC

push    edx                         ; WriteString needs EDX

mov     edx, OFFSET msgName

call    WriteString

mov     edx, esi

call    WriteString

call    Crlf

mov     edx, OFFSET msgReg

call    WriteString

mov     edx, edi

call    WriteString

call    Crlf

mov     edx, OFFSET msgCourse

call    WriteString

mov     edx, ebx

call    WriteString

call    Crlf

call    Crlf

pop     edx

ret                                 ; return address is on top of the stack

DisplayStudentInfo ENDP

END main

## Output

## Explanation

- Passing data: before each CALL, main loads ESI, EDI and EBX with the OFFSETs of that student's name, registration number and course title.

- CALL pushes the address of the next instruction onto the stack and jumps to the procedure.

- RET pops that address back into EIP, so execution continues with the instruction after each CALL.

- No duplication: the output instructions are written once inside the procedure. EDX is pushed and popped inside it so the caller's EDX is not changed.

# Task 5

Create a procedure CalculateResult that receives two unsigned integers in EAX and EBX and returns their sum, difference and product in registers. Test with several input pairs.

## Code

INCLUDE Irvine32.inc

.data

firstVals   DWORD 25, 120, 8            ; test inputs (first number)

secondVals  DWORD 7, 45, 20             ; test inputs (second number)

sumR        DWORD ?

diffR       DWORD ?

prodR       DWORD ?

msgCase     BYTE "Test ", 0

msgA        BYTE ":  A = ", 0

msgB        BYTE ",  B = ", 0

msgSum      BYTE "   Sum        (A + B) = ", 0

msgDiff     BYTE "   Difference (A - B) = ", 0

msgSigned   BYTE "   (signed view: ", 0

msgClose    BYTE ")", 0

msgProd     BYTE "   Product    (A * B) = ", 0

.code

main PROC

mov     esi, 0                      ; test index

mov     ecx, LENGTHOF firstVals     ; number of tests

NextTest:

; ---- show the inputs ----

mov     edx, OFFSET msgCase

call    WriteString

mov     eax, esi

inc     eax

call    WriteDec

mov     edx, OFFSET msgA

call    WriteString

mov     eax, firstVals[esi * TYPE firstVals]

call    WriteDec

mov     edx, OFFSET msgB

call    WriteString

mov     eax, secondVals[esi * TYPE secondVals]

call    WriteDec

call    Crlf

; ---- call the procedure ----

mov     eax, firstVals[esi * TYPE firstVals]    ; input 1

mov     ebx, secondVals[esi * TYPE secondVals]  ; input 2

call    CalculateResult

call    ShowResults

inc     esi                         ; ECX and ESI were preserved by the procedures

loop    NextTest

exit

main ENDP

;------------------------------------------------------------

; ShowResults

; Displays the three results returned by CalculateResult.

; Receives: EDI = sum, EDX = difference, EAX = product

;------------------------------------------------------------

ShowResults PROC

mov     sumR, edi

mov     diffR, edx

mov     prodR, eax

mov     edx, OFFSET msgSum

call    WriteString

mov     eax, sumR

call    WriteDec

call    Crlf

mov     edx, OFFSET msgDiff

call    WriteString

mov     eax, diffR

call    WriteDec                    ; unsigned result

mov     edx, OFFSET msgSigned

call    WriteString

call    WriteInt                    ; same bits as signed

mov     edx, OFFSET msgClose

call    WriteString

call    Crlf

mov     edx, OFFSET msgProd

call    WriteString

mov     eax, prodR

call    WriteDec

call    Crlf

call    Crlf

ret

ShowResults ENDP

;------------------------------------------------------------

; CalculateResult

; Calculates the sum, difference and product of two

; unsigned integers.

; Receives: EAX = first number (A), EBX = second number (B)

; Returns:  EDI = A + B

;           EDX = A - B

;           EAX = A * B (low 32 bits)

; Preserves: EBX, ECX, ESI

;------------------------------------------------------------

CalculateResult PROC

mov     edi, eax

add     edi, ebx                    ; EDI = sum

push    eax                         ; save A (MUL overwrites EAX and EDX)

mul     ebx                         ; EDX:EAX = A * B

pop     edx                         ; EDX = A

sub     edx, ebx                    ; EDX = A - B

ret

CalculateResult ENDP

END main

## Output

## Register Usage

| Register | Direction | Meaning |
| --- | --- | --- |
| EAX | Input | First number (A) |
| EBX | Input | Second number (B) — preserved |
| EDI | Output | Sum A + B |
| EDX | Output | Difference A − B |
| EAX | Output | Product A × B (low 32 bits) |
| ECX, ESI | Preserved | Not modified, so main can use them as the loop counter and index |

## Test Results

| A | B | Sum | Difference | Product |
| --- | --- | --- | --- | --- |
| 25 | 7 | 32 | 18 | 175 |
| 120 | 45 | 165 | 75 | 5400 |
| 8 | 20 | 28 | 4294967284 (−12 signed) | 160 |

MUL stores its result in EDX:EAX, which would destroy A. So A is pushed before MUL and popped into EDX afterwards to calculate A − B. In test 3 the first number is smaller than the second. The unsigned result wraps around to 4294967284, which is −12 when the same bits are read as a signed number.

# Task 6

Write a procedure AddStackNumbers that receives two 32-bit arguments pushed by the caller, returns their sum in EAX, and leaves the stack balanced so that RET uses the correct return address.

## Code

INCLUDE Irvine32.inc

.data

argA        SDWORD 150, -40, 1000       ; first argument of each test

argB        SDWORD 275, 90, -2500       ; second argument of each test

retAddr     DWORD ?                     ; return address saved by the procedure

argTemp     DWORD ?                     ; first argument removed from the stack

espBefore   DWORD ?

espAfter    DWORD ?

msgPlus     BYTE " + ", 0

msgEq       BYTE " = ", 0

msgBefore   BYTE "   ESP before PUSH = ", 0

msgAfter    BYTE "h   ESP after return = ", 0

msgBal      BYTE "h   (balanced)", 0

msgUnbal    BYTE "h   (NOT balanced)", 0

.code

main PROC

mov     esi, 0                      ; test index

mov     ecx, LENGTHOF argA          ; 3 tests

NextTest:

mov     espBefore, esp

push    argA[esi * TYPE argA]       ; first argument

push    argB[esi * TYPE argB]       ; second argument

call    AddStackNumbers             ; pushes the return address

mov     espAfter, esp               ; EAX = sum

call    ShowTest

inc     esi

loop    NextTest

exit

main ENDP

;------------------------------------------------------------

; ShowTest

; Displays "a + b = sum" and the ESP values for one test.

; Receives: EAX = sum, ESI = test index

;------------------------------------------------------------

ShowTest PROC

; ---- display "a + b = sum" ----

mov     ebx, eax                    ; keep the sum

mov     eax, argA[esi * TYPE argA]

call    WriteInt

mov     edx, OFFSET msgPlus

call    WriteString

mov     eax, argB[esi * TYPE argB]

call    WriteInt

mov     edx, OFFSET msgEq

call    WriteString

mov     eax, ebx

call    WriteInt

call    Crlf

; ---- display stack balance ----

mov     edx, OFFSET msgBefore

call    WriteString

mov     eax, espBefore

call    WriteHex

mov     edx, OFFSET msgAfter

call    WriteString

mov     eax, espAfter

call    WriteHex

mov     edx, OFFSET msgBal

cmp     eax, espBefore

je      ShowBal

mov     edx, OFFSET msgUnbal

ShowBal:

call    WriteString

call    Crlf

call    Crlf

ret

ShowTest ENDP

;------------------------------------------------------------

; AddStackNumbers

; Adds two 32-bit integers that the caller pushed on the stack.

; Stack on entry:  [ESP]   = return address

;                  [ESP+4] = second argument

;                  [ESP+8] = first argument

; Returns: EAX = first + second. Both arguments are removed,

;          so the stack is balanced when control returns.

;------------------------------------------------------------

AddStackNumbers PROC

pop     retAddr                     ; save the return address

pop     eax                         ; EAX = second argument

pop     argTemp                     ; first argument

add     eax, argTemp                ; EAX = sum

push    retAddr                     ; put the return address back on top

ret                                 ; RET pops the correct return address

AddStackNumbers ENDP

END main

## Output

## Stack Layout

| Moment | Stack (top first) | ESP |
| --- | --- | --- |
| Before PUSH | … | X |
| After PUSH first, PUSH second | second, first | X − 8 |
| After CALL (procedure entry) | return address, second, first | X − 12 |
| After POP retAddr, POP EAX, POP argTemp | (empty) | X |
| After PUSH retAddr | return address | X − 4 |
| After RET (back in main) | (empty) | X |

- Return address on top: right after CALL, the return address is at [ESP] and the arguments are below it. The procedure first pops the return address into the variable retAddr so it can reach the arguments.

- Arguments: the second argument was pushed last, so it is popped first into EAX; the first argument is popped into argTemp and added to EAX.

- Correct RET: the return address is pushed back so it is on top again. RET then pops the real return address, not an argument, into EIP.

- Balanced stack: the procedure removes both arguments, so ESP after returning equals ESP before the two PUSHes in all three tests.

| First | Second | EAX (WriteInt) |
| --- | --- | --- |
| 150 | 275 | +425 |
| −40 | 90 | +50 |
| 1000 | −2500 | −1500 |

# Task 7

Create AnalyzeValues, which receives two values in EAX and EBX, calculates their sum, performs another flag-changing operation, returns the sum in EAX and preserves the caller's EFLAGS with PUSHFD/POPFD.

## Code

INCLUDE Irvine32.inc

.data

msgCase1    BYTE "===== Case 1: CMP 7,7 (ZF=1) then AnalyzeValues(20, 30) =====", 0

msgCase2    BYTE "===== Case 2: CMP 1,2 (ZF=0, CF=1) then AnalyzeValues(45, -45) =====", 0

msgBefore   BYTE "Main, before CALL:", 0

msgInside   BYTE "Inside AnalyzeValues, after arithmetic (flags changed):", 0

msgAfter    BYTE "Main, after RET (caller flags restored, EAX = sum):", 0

.code

main PROC

xor     ecx, ecx                    ; clear registers so DumpRegs is easy to read

xor     esi, esi

xor     edi, edi

; ======================= Case 1 =======================

mov     edx, OFFSET msgCase1

call    WriteString

call    Crlf

mov     edx, OFFSET msgBefore

call    WriteString

mov     eax, 7

cmp     eax, 7                      ; recognizable state: ZF=1, CF=0, SF=0

mov     eax, 20                     ; MOV does not change flags

mov     ebx, 30

call    DumpRegs

call    AnalyzeValues               ; EAX = 50

pushfd                              ; keep the restored flags while printing

mov     edx, OFFSET msgAfter

call    WriteString

popfd

call    DumpRegs                    ; EAX = 00000032h, ZF=1 (caller's flag)

call    Crlf

; ======================= Case 2 =======================

mov     edx, OFFSET msgCase2

call    WriteString

call    Crlf

mov     edx, OFFSET msgBefore

call    WriteString

mov     eax, 1

cmp     eax, 2                      ; recognizable state: ZF=0, CF=1, SF=1

mov     eax, 45

mov     ebx, -45

call    DumpRegs

call    AnalyzeValues               ; EAX = 0

pushfd

mov     edx, OFFSET msgAfter

call    WriteString

popfd

call    DumpRegs                    ; EAX = 0 but ZF=0 (caller's flag)

exit

main ENDP

;------------------------------------------------------------

; AnalyzeValues

; Adds two values and also computes their difference, which

; changes the status flags. The caller's EFLAGS are preserved.

; Receives: EAX = first value, EBX = second value

; Returns:  EAX = EAX + EBX

; Preserves: EFLAGS, EBX, ECX, EDX

;------------------------------------------------------------

AnalyzeValues PROC

pushfd                              ; save the caller's flags first

push    ecx

push    edx

mov     edx, OFFSET msgInside

call    WriteString

mov     ecx, eax

sub     ecx, ebx                    ; ECX = first - second (changes flags)

add     eax, ebx                    ; EAX = sum (changes flags again)

call    DumpRegs                    ; shows the procedure's own flags

pop     edx

pop     ecx

popfd                               ; restore caller's flags just before RET

ret                                 ; EAX keeps the sum: POPFD restores flags only

AnalyzeValues ENDP

END main

## Output

## Expected vs Actual

| Case | Caller flags before CALL | Flags inside procedure | EAX after RET | ZF after RET |
| --- | --- | --- | --- | --- |
| 1: CMP 7,7 then (20, 30) | ZF=1, CF=0, SF=0 | ZF=0, CF=0 (50 ≠ 0) | 00000032h (50) | 1 (caller's) |
| 2: CMP 1,2 then (45, −45) | ZF=0, CF=1, SF=1 | ZF=1, CF=1 (sum = 0) | 00000000h (0) | 0 (caller's) |

- Saving the flags: PUSHFD is the first instruction of the procedure and POPFD is the last one before RET, so every flag change inside the procedure is undone before returning.

- Additional operation: SUB ECX, EBX computes the difference (20 − 30 = −10 in case 1, 45 − (−45) = 90 in case 2) and ADD EAX, EBX computes the sum; both change the flags, as the "inside" DumpRegs shows.

- EAX is kept: POPFD restores only EFLAGS, so the sum stays in EAX. In case 2 EAX is 0 but ZF is 0 after return, which proves ZF comes from the caller, not from the sum.

- Stack correctness: the procedure pushes EFLAGS, ECX and EDX and pops them in reverse order, so the return address is on top when RET executes. ESP after each return equals ESP before the call, and ECX/EDX are restored as well.